IEEE RFID Lockbox Instructions

Electronics:

For the electronics on this project there are a few different parts. Here is the main wiring diagram for the whole project.

- Servo

The servo has three wires that need connected. The orange wire is the control wire and is connected to D7 on the Arduino Nano. The Red wire is +5V and the brown wire is GND. To test the servo out and see the code that is required to run it, in the Arduino IDE go to File > Examples > Servo >

- LCD

To test the LCD and operate it there are 4 different wires that need connected to the Arduino Nano. Reference the wiring diagram to see what is connected to what. The pins on the LCD are labeled. To run the LCD you will need to install the LiquidCrystal I2C library in the Arduino IDE. To test the LCD select File > Examples > LiquidCrystal I2C > HelloWorld.

- RFID

For soldering on the header pins on the RFID make sure you solder the 90 degree pins. To operate the RFID, you will need to install the MFRC522 library in the Arduino IDE. To run an example code and learn how it works open File > Examples > MFRC522 > ReadNUID. Wire up all 8 pins of the RFID read according to the wiring diagram and upload the code to you Arduino nano. Here is a link to more info https://werdna-b.github.io/mkdocs-test/rfid_docs/

- LED Light Strip

We will put instructions here as soon as possible

Software:

- Go to NNU-IEEE-Lockbox/source at features/reformat · DeltaDDX/NNU-IEEE-Lockbox

- Download both “lock_box_v1.ino” and “low-level-functions.h”

- Open “lock_box_v1.ino” and select “ok” when asked to create a new folder.

- On the left side of the Arduino IDE, click the three dots on the sketch and select “Open Sketch Folder”

- Place “low-level-functions.h” in that folder.

- You’re done! Feel free to mess around with the functions in the main project folder and add your own logic to when they are called!

Mechanical:

The box consists of three basic systems: The shell, which hopefully holds your stuff, the lid, which contains all the electrical components, and the latch, which keeps the box shut when necessary.

Shell:

1. These are sockets for individual LEDs. They can be used for things like power level monitoring, flashing when a card is near, successful unlock, error, etc. Multiple colors are available. The wires for them route through Feature 3, the LED strip slot.

2. This is the catch for the latch. It holds the latch shut.

3. This is an inset for the LED light strand in the diffusion strip. Wires can also be run through it.

4. This is the hinge socket for the cap screw that is functioning as our hinge.

5. This is a slot that the power shutoff switch plugs into. It’s a basic rocker switch.

6. This is the USB-C port for recharging the battery.

Lid:

1. This is a nice slot in the Inset plate {Feature 5}, that lets you hide the circuit board deep inside.

2. This is a spring holder for the springback system that auto-springs the box open.

3. This is the other end of the hinge.

4. This inset holds the servo.

5. This is the plate that screws into the lid itself to keep everything contained.

6. This holds the buzzer in place.

7. Whoa! These are for screws to hold the thing in place! Whoa! (Live long and prosper, young padawans)

8. This little thingy is an inset for the LCD that is visible from the top of the lid.

Latch

- This is the latch.

- This keeps the latch from going sideways or getting jammed or otherwise misbehaving.

- This is the double-barreled holder for the springs that force the latch back open.

- This little tail is for the cabling that goes to the servo to provide a mechanical connection betwixt the two assemblies

Assembly

Hitch all the pieces together, and integrate the components as indicated in the pictures. Then wire it all up according to the schematic. After that is done, put a bolt everywhere it looks like one is needed.

Slicing

To print the parts, each model has to be run through a slicer. The slicer only knows what to do with .stl files, which are included in the github respository. If you modify the box design provided, you’ll want to export that model to .stl so your changes are reflected.

The slicer most people use is called OrcaSlicer. Cura also works if you’re feeling anti-Orca.

When setting the slicing options, you don’t want to use 100% infill! This will take forever to print (On the order of a week) and will use more filament that is available. In other words, it ain’t gonna work. Thus, it is recommended that an infil rate of 5-10% is used on the main box, ~10% on the lid plate and the lid, and 10-15% on the latch pieces.

Here the filament diameter is set to .24mm. This means it will push out more filament, which makes the print process go faster, but it also won’t be quite as precise. For the box, this is a good setting. For the others, .20 or .18mm would be better.

Support will be necessary on the box. Tree is easier to deal with than normal, so I would reccomend using that.

The other main strength settings are wall loops, top shell layers, and bottom shell layers. For the optimal strength/time ratios, set all these values to 3.  This is also the menu where you set infill density.